OUTLINE FOR QUESTION 1 PRESENTATION

QUESTION 1:

- Where and when does the IMPROVE data base show high OC/EC ratios that might indicate large contributions of secondary organic aerosol to light extinction?

- Review geographic extent of IMPROVE network

- Monitoring methodologies used in the IMPROVE network with an emphasis on Thermal Optical Reflectance (TOR) carbon measurement.

- General spatial trends in elemental and organic carbon.

- Literature review of OC/EC source profile measurements using TOR analytic technique.

- Review of OC/EC measurements found in the IMPROVE monitoring network

  - Seasonality of OC, EC, and OC/EC ratios – If biogenic emissions increase during summer months as well as photochemical processes enhanced one might expect seasonal variability in OC and OC/EC ratios.

  - Review other mechanisms for seasonal variability in carbon based measurements such as transport and mixing depths.  Do we see evidence of these effects in the IMPROVE network.

  - Examine OC, EC, and OC/EC ratios in geographic regions known to have significant wild and prescribed fire contributions.

  - Examine OC, EC, and OC/EC ratios in urban areas such as Washington DC.

- From a qualitative perspective rural regions of the eastern United States may have approximately enhanced OC/EC ratios of about a factor of 3-4 suggesting that secondary organics are about a factor of 3-4 greater during summer season compared to winter.

- Other areas of the United States such as the west coast have OC/EC summer vs winter enhancements that are  almost negligible.

- The north central US appear to be dominated by fire emissions in that the OC/EC ratios are near that for fire related activity and show less seasonal variability than typical eastern sites.

- Examine long term trends in OC, EC, and OC/EC ratios.

- What role does changes in TOR analytic methodology and artifact corrections play in observed trends in OC, EC and OC/EC ratios?